# FAITH WEAR APPAREL (FWA)

## PHASE 1 IMPLEMENTATION – MANUFACTURING SETUP & BOM CONFIGURATION

Document Version: 1.0
 Date: 19 December 2025
 Database: terraforegfwa (Odoo Online)
 Module: Manufacturing > Configuration

## EXECUTIVE SUMMARY

This document details FWA's manufacturing setup including:

- 2 Current Work Centers (CMT Sandy, Embroidery Dave)

- 1 Recommended Work Center (Printing – to be added)

- 2 Current Operations (CMT Cutting/Sewing, Embroidery)

- 1 Recommended Operation (Printing – to be added)

- Actual BOM Example (Golf Shirt Regular Men CMT)

- Manufacturing Workflow (From component picking to finished goods)

This configuration enables FWA's multi-step manufacturing and subcontracting operations with clear operation tracking and resource management.

# PART 1: WORK CENTERS

Work Centers represent production facilities or vendor service centers where manufacturing operations are performed.

## 1.1 Work Centers Overview

Current Work Centers: 2
 Recommended Work Centers: 3 (add Printing)

## 1.2 Work Center 1: CMT Sandy

Cut, Make, Trim (CMT) Assembly Operation

| Field | Value |
| --- | --- |
| Work Center Name | CMT Sandy |
| Work Center Type | Subcontracting Partner (Local Zimbabwe) |
| Status | Normal ✅ |
| OEE Target | 90% |
| Capacity Category | Garment Assembly |

Responsibilities:

- Fabric cutting (from supplied materials)

- Garment assembly (sewing)

- Attaching labels and trims

- Quality inspection at CMT level

- Packaging for return shipment

Typical Operations:

- CMT Cutting/Sewing (60 cycles per unit)

Output:

- Assembled garments ready for decoration (printing/embroidery)

- Examples: Golf Shirts, T-Shirts, Trousers

Lead Time: 3-5 days (depending on volume)

Quality Metrics:

- Seam quality (stitching consistency)

- Fit accuracy (sizing)

- Label placement

- Defect rate target: <2%

## 1.3 Work Center 2: Embroidery Dave

Embroidery & Decoration Operation

| Field | Value |
| --- | --- |
| Work Center Name | Embroidery Dave |
| Work Center Type | Subcontracting Partner (Speciality) |
| Status | Normal ✅ |
| OEE Target | 90% |
| Capacity Category | Embroidery & Finishing |

Responsibilities:

- Embroidery (logos, company names, designs)

- Thread color matching

- Embroidery placement accuracy

- Quality control on embroidery

- Final finishing touches

Typical Operations:

- Embroidery (60 cycles per unit)

Input:

- Assembled garments from CMT (or internal production)

- Embroidery thread and design specifications

Output:

- Embroidered finished goods ready for shipment

Lead Time: 2-3 days

Quality Metrics:

- Thread quality and color accuracy

- Embroidery placement (accuracy within 5mm)

- Knot security

- Defect rate target: <1%

## 1.4 Work Center 3: PRINTING (RECOMMENDED TO ADD)

Print & Screen Printing Operation

Recommendation: Add this work center to handle screen printing and digital printing operations.

| Field | Recommended Value |
| --- | --- |
| Work Center Name | Printing [Vendor Name] |
| Work Center Type | Subcontracting Partner (Print Specialist) |
| Status | Normal (to be created) |
| OEE Target | 90% |
| Capacity Category | Decoration / Printing |

Responsibilities:

- Screen printing (logos, designs on garments)

- Digital printing (custom graphics)

- Color matching

- Print placement accuracy

- Quality control on prints

- Final curing/heat-setting

Typical Operations:

- Printing (to be defined: cycles per unit)

Input:

- Assembled garments from CMT

- Print files, color specifications

- Customer branding requirements

Output:

- Printed garments ready for embroidery or shipment

Lead Time: 2-3 days

Quality Metrics:

- Print clarity and color accuracy

- Ink saturation

- Print placement (accuracy within 10mm)

- Wash fastness (color doesn't fade)

- Defect rate target: <1%

## 1.5 Work Center Decision Matrix

Selecting Between Printing & Embroidery:

| Operation | Use Case | Lead Time | Cost |
| --- | --- | --- | --- |
| Printing (Screen/Digital) | Large logos, photo prints, complex designs, bulk orders | 2-3 days | Lower per unit (good for bulk) |
| Embroidery (Thread) | Premium look, small logos, corporate branding, durability | 2-3 days | Higher per unit (premium) |
| Both | Full branding (print + embroidery combined) | 4-5 days | Combined cost |

Recommendation for FWA: Maintain both capabilities to offer customers choice.

# PART 2: OPERATIONS & WORK CENTER USAGE

Operations define the specific manufacturing steps and link them to work centers.

## 2.1 Operations Overview

Current Operations: 2
 Recommended Operations: 3 (add Printing)

## 2.2 Operation 1: CMT Cutting/Sewing

Garment Assembly Operation

| Field | Value |
| --- | --- |
| Operation Name | CMT Cutting/Sewing |
| Operation Type | Subcontracting (Send to CMT vendor) |
| Work Center | CMT Sandy |
| BoM Reference | GSRM BOM1 (CMT): Golf Shirt Regular Men (CMT) |
| Cycles | 60 |
| Instructions | 0 (Standard operation, no special instructions) |

Process Steps:

- Receive fabric, trims, and labels at CMT facility

- Cut fabric pieces according to size specifications

- Sew pieces together (seams, sleeves, neckline)

- Attach labels (main brand label, size label, care label)

- Attach buttons/closures if applicable

- Quality check (stitching, fit, alignment)

- Package for return shipment

Quality Checkpoints:

- ✓ Seam straightness

- ✓ Stitch tension consistency

- ✓ Label placement accuracy

- ✓ Button/closure security

- ✓ Size compliance

Output: Assembly-complete garments ready for decoration

## 2.3 Operation 2: Embroidery

Embroidery & Decoration Operation

| Field | Value |
| --- | --- |
| Operation Name | Embroidery |
| Operation Type | Decoration (External vendor) |
| Work Center | Embroidery Dave |
| BoM Reference | GSRM BOM1 (CMT): Golf Shirt Regular Men (CMT) |
| Cycles | 60 |
| Instructions | 0 |

Process Steps:

- Receive assembled garments from CMT

- Hoop garment on embroidery machine

- Load design file (customer logo, company name)

- Set thread colors and machine parameters

- Run embroidery machine

- Trim thread tails

- Quality check (thread knots, color accuracy, placement)

- Remove from hoop and inspect

- Final pressing/finishing

Quality Checkpoints:

- ✓ Design alignment (within 5mm)

- ✓ Thread color accuracy

- ✓ Knot security

- ✓ Stitch density

- ✓ No missed stitches

Output: Embroidered finished goods ready for shipment

## 2.4 Operation 3: PRINTING (RECOMMENDED TO ADD)

Screen Printing & Digital Printing Operation

Recommendation: Add this operation to support printing orders.

| Field | Recommended Value |
| --- | --- |
| Operation Name | Printing |
| Operation Type | Decoration (External vendor) |
| Work Center | Printing [Vendor Name] – To be created |
| BoM Reference | Applicable to BOMs requiring printing |
| Cycles | 45 (estimated – adjust based on actual) |
| Instructions | 0 (or add if special requirements) |

Process Steps:

- Receive assembled garments from CMT

- Prepare print files (design, color separations)

- Set up screens (for screen printing) or digital printer

- Test print on sample

- Adjust color/placement as needed

- Production run

- Cure/heat-set print (if needed)

- Quality check (color, clarity, placement)

- Cool and inspect

- Package

Quality Checkpoints:

- ✓ Print clarity and sharpness

- ✓ Color accuracy (match to design specification)

- ✓ Print placement (within 10mm)

- ✓ Ink coverage

- ✓ Wash fastness (color durability)

Output: Printed garments ready for embroidery (if combined) or shipment

## 2.5 Work Center Usage Summary

### Current Work Center Usage

| Operation | Work Center | Cycles | BoM |
| --- | --- | --- | --- |
| CMT Cutting/Sewing | CMT Sandy | 60 | GSRM BOM1 (CMT): Golf Shirt Regular Men (CMT) |
| Embroidery | Embroidery Dave | 60 | GSRM BOM1 (CMT): Golf Shirt Regular Men (CMT) |

### Recommended Addition

| Operation | Work Center | Cycles | BoM |
| --- | --- | --- | --- |
| Printing | Printing [Vendor] | 45 | Multiple BOMs (as applicable) |

# PART 3: BILL OF MATERIALS (BOM) EXAMPLE

This section details the actual BOM configured for Golf Shirt Regular Men (CMT).

## 3.1 BOM Overview

| Field | Value |
| --- | --- |
| BoM Name | Golf Shirt Regular Men (CMT) |
| BoM Reference | GSRM BOM1 (CMT) |
| Product | Golf Shirt Regular Men (CMT) |
| Product Variant | (None – parent product) |
| BoM Type | Manufacture this product ✅ |
| Quantity | 1.00 Units |
| Alternative BoM | (None) |
| Cost Structure | All-in (includes labor, materials, services) |

## 3.2 Components Tab

### Raw Materials & Services in BOM

Component 1: Fabric La Coste Knit (Color Variants)

| Field | Black Variant | Red Variant | White Variant |
| --- | --- | --- | --- |
| Component Name | Fabric La Coste Knit Black | Fabric La Coste Knit Red | Fabric La Coste Knit White |
| Quantity | 1.50 | 1.50 | 1.50 |
| Unit | meters (mts) | meters (mts) | meters (mts) |
| Apply on Variants | color: Black | color: Red | color: White |
| Consumed in Operation | CMT Cutting/Sewing | CMT Cutting/Sewing | CMT Cutting/Sewing |

Purpose: Primary fabric for garment body
 Fabric Type: Piqué knit (common for polo/golf shirts)
 Weight: 200gsm (typical for corporate wear)
 Supplier: Fabric supplier (local or imported)

Variant Logic:

- When Black shirt is ordered → use Black fabric

- When Red shirt is ordered → use Red fabric

- When White shirt is ordered → use White fabric

Component 2: Clothing Label (Top)

| Field | Value |
| --- | --- |
| Component Name | Clothing Label (Top) |
| Quantity | 1.00 |
| Unit | Units |
| Apply on Variants | (None – all shirts) |
| Consumed in Operation | CMT Cutting/Sewing |
| Product Type | Service / Material |

Purpose: Main brand label (sewn onto shirt)
 Content: "Faith Wear Apparel" branding
 Placement: Typically on side seam or bottom
 Supplier: Label supplier (local)

Component 3: CMT Service

| Field | Value |
| --- | --- |
| Component Name | CMT Service |
| Quantity | 1.00 |
| Unit | Units |
| Apply on Variants | (None – all shirts) |
| Consumed in Operation | CMT Cutting/Sewing |
| Product Type | Service (not physical material) |

Purpose: Subcontracting fee for CMT Sandy (assembly labor)
 Includes: Cutting, sewing, label attachment, QC at CMT level
 Cost: Variable (per unit, bulk discounts may apply)
 Vendor: CMT Sandy

Cost Capture: This line captures the labor cost of CMT assembly so management can see:

- Material cost (fabrics, labels)

- Service cost (CMT labor)

- Total manufacturing cost per unit

Component 4: Embroidery Service

| Field | Value |
| --- | --- |
| Component Name | Embroidery Service |
| Quantity | 1.00 |
| Unit | Units |
| Apply on Variants | (Potentially – can limit to certain colors) |
| Consumed in Operation | Embroidery |
| Product Type | Service (not physical material) |

Purpose: Subcontracting fee for Embroidery Dave (embroidery labor)
 Includes: Hooping, design placement, embroidery stitching, QC
 Cost: Variable (per unit, based on design complexity)
 Vendor: Embroidery Dave

Cost Capture: This line captures the embroidery service cost so management can see margin impact of decoration.

## 3.3 Components Summary Table

| # | Component | Qty | Unit | Variant Logic | Operation |
| --- | --- | --- | --- | --- | --- |
| 1 | Fabric La Coste Knit (Black, Red, White) | 1.50 | mts | color-based | CMT Cutting/Sewing |
| 2 | Clothing Label (Top) | 1.00 | Units | All | CMT Cutting/Sewing |
| 3 | CMT Service | 1.00 | Units | All | CMT Cutting/Sewing |
| 4 | Embroidery Service | 1.00 | Units | All | Embroidery |

Total Material Cost: Sum of fabric, label, and trim costs
 Total Labor Cost: CMT Service + Embroidery Service
 Total BoM Cost: Materials + Labor

## 3.4 Operations Tab

### Manufacturing Operations in BoM

| # | Operation | Work Center | Cycles | Duration | Sequence |
| --- | --- | --- | --- | --- | --- |
| 1 | CMT Cutting/Sewing | CMT Sandy | 60 | 3-5 days | First (assembly) |
| 2 | Embroidery | Embroidery Dave | 60 | 2-3 days | Second (decoration) |

Workflow:

1. CMT Cutting/Sewing (CMT Sandy)

Input: Fabrics, label, buttons

Output: Assembled shirt

Time: 3-5 days

↓

2. Embroidery (Embroidery Dave)

Input: Assembled shirt + embroidery specs

Output: Finished embroidered shirt

Time: 2-3 days

↓

Final: Quality check → Stock → Delivery

## 3.5 Manufacturing Instructions

Currently: 0 instructions

Consideration for Enhancement: If special manufacturing instructions are needed (e.g., "Use navy thread for stitching" or "Embroidery must be placed 2cm from seam"), these can be added to provide guidance to CMT and embroidery vendors.

# PART 4: RECOMMENDED BOM ENHANCEMENTS

## 4.1 Add Printing Service to BOM

Current BOM Components: 4
 Recommended Addition: Printing Service

Scenario: Customer orders Golf Shirts with both printing (logo) and embroidery (company name).

Enhanced BoM Structure:

| # | Component | Qty | Unit | Operation |
| --- | --- | --- | --- | --- |
| 1 | Fabric La Coste Knit (variants) | 1.50 | mts | CMT Cutting/Sewing |
| 2 | Clothing Label (Top) | 1.00 | Units | CMT Cutting/Sewing |
| 3 | CMT Service | 1.00 | Units | CMT Cutting/Sewing |
| 4 | Printing Service | 1.00 | Units | Printing ✅ (NEW) |
| 5 | Embroidery Service | 1.00 | Units | Embroidery |

Modified Workflow:

1. CMT Cutting/Sewing (CMT Sandy) → Assembled shirt

↓

2. Printing (Printing vendor) → Printed shirt

↓

3. Embroidery (Embroidery Dave) → Finished shirt

↓

Final: QC → Stock → Delivery

Cost Impact Example:

- Fabric + Label: $3.50

- CMT Service: $2.00

- Printing Service: $1.50 (NEW)

- Embroidery Service: $2.00

- Total BOM Cost: $9.00

## 4.2 Create Product Variant BOMs

Current: 1 BOM for all Golf Shirt colors (uses variant-specific fabric lines)

Alternative Approach: Create separate BOMs for each variant (if desired)

| BoM | Product | Variant | Fabric |
| --- | --- | --- | --- |
| GSRM BOM1 (CMT) | Golf Shirt Regular Men | (All colors) | Variant-specific fabric |
| GSRM BOM1-BLK | Golf Shirt Regular Men | Black | Black fabric only |
| GSRM BOM1-RED | Golf Shirt Regular Men | Red | Red fabric only |
| GSRM BOM1-WHT | Golf Shirt Regular Men | White | White fabric only |

Current Approach (Recommended): Use the single BOM with variant-specific lines → simpler management, works well for FWA.

# PART 5: BOM SETUP WORKFLOW

## 5.1 How to Create a BoM in Odoo

Step-by-Step:

- Navigate: Manufacturing > Products > Bills of Materials > New

- Header Section:

  - Product: Select finished good (e.g., Golf Shirt Regular Men)

  - BoM Type: "Manufacture this product" (for internally made products)

  - Quantity: 1.00 (how many units this BoM produces)

- Components Tab:

  - Add raw materials (fabrics, buttons, labels)

  - Add service lines (CMT Service, Embroidery Service, Printing Service)

  - Set quantities (e.g., 1.50 meters fabric)

  - Set unit of measure (mts for fabric, Units for services)

  - Link to variants if applicable (e.g., "color: Black" for black fabric)

  - Assign operation (e.g., "CMT Cutting/Sewing" for fabric)

- Operations Tab:

  - Add manufacturing operations (CMT, Embroidery, Printing)

  - Assign work center for each operation

  - Set cycles (default 60)

  - Order operations in sequence

- Save & Confirm

- Use in Manufacturing Orders:

  - When Sales Order is confirmed → Manufacturing Order created

  - Manufacturing Order pulls components from this BoM

  - Work orders created for each operation

## 5.2 BoM Components Mapping

For Golf Shirt Regular Men (CMT) Example:

Golf Shirt (finished product)

│

├─ Materials (picked from stock)

│  ├─ Fabric La Coste Knit (1.50 mts) → color-based variant

│  └─ Clothing Label (1.00 unit)

│

├─ Services (subcontracting)

│  ├─ CMT Service (1.00 unit) → Operation: CMT Cutting/Sewing

│  ├─ Printing Service (1.00 unit) → Operation: Printing (NEW)

│  └─ Embroidery Service (1.00 unit) → Operation: Embroidery

│

└─ Operations (in sequence)

├─ 1. CMT Cutting/Sewing (Work Center: CMT Sandy, 60 cycles)

├─ 2. Printing (Work Center: Printing vendor, 45 cycles) [NEW]

└─ 3. Embroidery (Work Center: Embroidery Dave, 60 cycles)

# PART 6: MANUFACTURING FLOW & WORK ORDER CREATION

## 6.1 From Sales Order to Manufacturing Orders

Flow:

Customer Sales Order (Golf Shirts, custom colors)

↓

Confirm SO (MTO route: Replenish on Order)

↓

Check stock: Insufficient finished goods

↓

Trigger Manufacturing Order

↓

Manufacturing Order pulls BoM: GSRM BOM1 (CMT)

↓

Component Requirements:

- 1.50m fabric (color-based)

- 1 label

- 1 CMT Service

- 1 Printing Service (if applicable)

- 1 Embroidery Service

↓

Create Work Orders:

1. CMT Cutting/Sewing (CMT Sandy) → Vendor order

2. Printing (Printing vendor) → Vendor order [if added]

3. Embroidery (Embroidery Dave) → Vendor order

↓

Subcontracting POs sent to vendors

↓

Vendors complete work in sequence

↓

Return to Production location

↓

QC Check & Final Assembly

↓

Move to WHFWA/Stock (Finished Goods)

↓

Delivery Order created

↓

Ship to Customer

## 6.2 Work Order Details Example

Work Order 1: CMT Cutting/Sewing

| Field | Value |
| --- | --- |
| Work Order Number | MO-001-001 (Manufacturing Order 001, Operation 1) |
| Operation | CMT Cutting/Sewing |
| Work Center | CMT Sandy |
| Quantity | 500 units |
| Status | Waiting for materials |
| Planned Date | 2025-12-22 |
| Expected Duration | 3-5 days |

Components Needed:

- 750m fabric (500 × 1.5m)

- 500 labels

- 500 CMT Service units

Subcontracting PO Created:

- Vendor: CMT Sandy

- Materials: Fabrics, labels, buttons

- Expected return: 2025-12-25

Work Order 2: Printing (NEW)

| Field | Value |
| --- | --- |
| Work Order Number | MO-001-002 |
| Operation | Printing |
| Work Center | Printing [Vendor] |
| Quantity | 500 units |
| Status | Waiting for CMT completion |
| Planned Date | 2025-12-25 |
| Expected Duration | 2-3 days |

Input: Assembled shirts from CMT
 Service: Logo printing (custom design)
 Output: Printed shirts ready for embroidery

Work Order 3: Embroidery

| Field | Value |
| --- | --- |
| Work Order Number | MO-001-003 |
| Operation | Embroidery |
| Work Center | Embroidery Dave |
| Quantity | 500 units |
| Status | Waiting for printing completion |
| Planned Date | 2025-12-27 |
| Expected Duration | 2-3 days |

Input: Printed shirts from Printing vendor
 Service: Company name embroidery
 Output: Finished embroidered shirts → stock

# PART 7: CREATING THE PRINTING OPERATION & WORK CENTER

## 7.1 Step-by-Step: Add Printing Work Center

In Odoo, go to: Manufacturing > Configuration > Work Centers > New

Fill in:

| Field | Value |
| --- | --- |
| Name | Printing [Vendor Name] |
| Resource Type | Human |
| Capacity | 1.0 (or adjust based on vendor) |
| Time Efficiency | 100% |
| Default Waiting Duration | 0.0 hours |
| Default Working Duration | 1.0 hour |
| Sequence | 2 (between CMT and Embroidery) |
| Status | Normal ✅ |
| OEE Target | 90% |

Optional Fields:

- Color: Assign a color for visual distinction

- Description: "External print vendor for logo and custom graphics"

Save.

## 7.2 Step-by-Step: Add Printing Operation to BOM

Go to: Manufacturing > Bills of Materials > GSRM BOM1 (CMT)

Operations Tab > Add a Line:

| Field | Value |
| --- | --- |
| Operation | Printing |
| Work Center | Printing [Vendor Name] |
| Sequence | 2 (between CMT=1 and Embroidery=3) |
| Cycles | 45 |
| Instructions | (Leave blank or add: "Custom logo design per customer specification") |

Save.

## 7.3 Step-by-Step: Add Printing Service Component to BOM

Go to: Manufacturing > Bills of Materials > GSRM BOM1 (CMT)

Components Tab > Add a Line:

| Field | Value |
| --- | --- |
| Product | Printing Service |
| Quantity | 1.00 |
| Unit of Measure | Units |
| Apply on Variants | (Leave blank – applies to all) |
| Consumed in Operation | Printing |

Note: You may need to create "Printing Service" as a product first (Manufacturing > Products > Services).

Save.

## 7.4 Update Manufacturing Order to Use Printing

Once Printing is added:

- New Manufacturing Orders automatically include Printing Service

- Existing BOMs need to be updated if you want to add printing retroactively

- Create variant-specific BOMs if some styles print and others don't

# PART 8: PRINTING VENDOR SELECTION

## 8.1 Finding & Evaluating Print Vendors

For Screen Printing (bulk orders, durable):

- Local Zimbabwe print shops

- South Africa suppliers (if importing)

- Quality: 3-5 day turnaround, ~$1-3 per unit

For Digital Printing (custom, small batches, premium):

- On-demand print services

- Quality: 2-3 day turnaround, ~$1.50-3 per unit

Recommendation for FWA:

- Use 1-2 trusted printing vendors

- Set up as Subcontracting Partners (same as CMT/Embroidery)

- Negotiate volume discounts

- Conduct quality audits regularly

# PART 9: MANUFACTURING CONFIGURATION CHECKLIST

Use this checklist to verify all manufacturing settings are configured for go-live:

## Work Centers

- [ ] CMT Sandy configured

- [ ] Embroidery Dave configured

- [ ] Printing [Vendor] configured (RECOMMENDED TO ADD)

## Operations

- [ ] CMT Cutting/Sewing linked to CMT Sandy

- [ ] Embroidery linked to Embroidery Dave

- [ ] Printing linked to Printing work center (TO ADD)

## BOM: Golf Shirt Regular Men (CMT)

- [ ] Product: Golf Shirt Regular Men (CMT)

- [ ] BoM Type: Manufacture this product ✅

- [ ] Quantity: 1.00 Units

- [ ] Components:

  - [ ] Fabric La Coste Knit (color variants)

  - [ ] Clothing Label (Top)

  - [ ] CMT Service

  - [ ] Printing Service (RECOMMENDED TO ADD)

  - [ ] Embroidery Service

## Operations in BoM

- [ ] CMT Cutting/Sewing (Sequence 1)

- [ ] Printing (Sequence 2) – TO ADD

- [ ] Embroidery (Sequence 3)

## Manufacturing Workflow Testing

- [ ] Create test SO: 100 Golf Shirts, custom colors

- [ ] Verify Manufacturing Order created

- [ ] Verify component requirements calculated correctly

- [ ] Verify work orders created for each operation

- [ ] Verify subcontracting POs sent to vendors

- [ ] Verify cost tracking (material + labor)

# PART 10: ADDITIONAL BOMS TO CONFIGURE

Beyond Golf Shirt, consider these additional BOMs:

| Product | BoM Name | Components | Operations |
| --- | --- | --- | --- |
| T-Shirt | TS BOM1 | Fabric (2m), Label, CMT, Printing/Embroidery | CMT → Printing/Embroidery |
| Trousers | TR BOM1 | Fabric (2.5m), Zipper, Buttons, Label, CMT | CMT → QC |
| Hoodies | HD BOM1 | Fabric (3m), Drawstring, Label, CMT | CMT → Embroidery |
| Caps | CP BOM1 | Fabric (0.5m), Visor material, Label, Cap Assembly | Assembly → Embroidery |

Each BoM should define:

- Raw material components

- Subcontracting services (CMT, Printing, Embroidery)

- Operations sequence

- Quantities per size/variant

# PART 11: LABOR & SERVICE COSTING

## 11.1 Understanding Service Components in BoM

FWA uses Service lines in the BoM to capture labor costs:

Material Cost:

Fabric (1.50m × $2.50/m) = $3.75

Label (1 × $0.25) = $0.25

Subtotal Materials = $4.00

Service/Labor Cost:

CMT Assembly (1 × $2.00) = $2.00

Printing (1 × $1.50) = $1.50

Embroidery (1 × $2.00) = $2.00

Subtotal Labor = $5.50

Total BoM Cost = $9.50

## 11.2 Landed Cost Tracking

Landed Cost Features Available:

- Track additional costs (customs, transport, import duties)

- Allocate landed costs to products (for accurate COGS)

- Visibility for margin calculation

For FWA:

- Include CMT/Printing/Embroidery as "service costs"

- Include import duties (if applicable for imported fabrics)

- Include transport costs to/from vendors

# APPENDIX A: MANUFACTURING CONFIGURATION SUMMARY

| Component | Current | Recommended |
| --- | --- | --- |
| Work Centers | 2 (CMT Sandy, Embroidery Dave) | 3 (Add: Printing Vendor) |
| Operations | 2 (CMT, Embroidery) | 3 (Add: Printing) |
| BOMs Configured | 1 (Golf Shirt Regular Men CMT) | 5+ (all product types) |
| Components per BoM | 4 | 5+ (with printing service) |
| Service Lines | Yes (CMT, Embroidery) | Yes (+ Printing) |
| Variant Handling | Variant-specific component lines ✅ | Continue this approach |
| Cost Tracking | Material + Labor ✅ | Enhanced with Printing |

# APPENDIX B: EXAMPLE: GOLF SHIRT BOM WITH PRINTING ADDED

Complete BoM Structure (Enhanced):

GSRM BOM1 (CMT): Golf Shirt Regular Men (CMT)

Reference: GSRM BOM1 (CMT)

Quantity: 1.00 Units

BoM Type: Manufacture this product

COMPONENTS:

1. Fabric La Coste Knit Black (1.50 mts) - color: Black → Operation: CMT Cutting/Sewing

2. Fabric La Coste Knit Red (1.50 mts) - color: Red → Operation: CMT Cutting/Sewing

3. Fabric La Coste Knit White (1.50 mts) - color: White → Operation: CMT Cutting/Sewing

4. Clothing Label (Top) (1.00 Units) → Operation: CMT Cutting/Sewing

5. CMT Service (1.00 Units) → Operation: CMT Cutting/Sewing

6. Printing Service (1.00 Units) → Operation: Printing [NEW]

7. Embroidery Service (1.00 Units) → Operation: Embroidery

OPERATIONS:

1. CMT Cutting/Sewing - Work Center: CMT Sandy, Cycles: 60

2. Printing - Work Center: Printing [Vendor], Cycles: 45 [NEW]

3. Embroidery - Work Center: Embroidery Dave, Cycles: 60

# PART 12: NEXT STEPS FOR GO-LIVE

## Pre-Go-Live (19-22 Dec)

- [ ] Confirm printing vendor (if adding printing to workflow)

- [ ] Create Printing work center in Odoo

- [ ] Add Printing operation to relevant BOMs

- [ ] Create "Printing Service" product (if not exists)

- [ ] Test Manufacturing Order with Printing operation

- [ ] Verify work order sequence (CMT → Printing → Embroidery → QC)

- [ ] Verify subcontracting POs sent correctly to printing vendor

## Go-Live (22 Dec)

- ✅ Manufacturing Orders auto-created from Sales Orders

- ✅ Work orders created in correct sequence

- ✅ Subcontracting POs sent to vendors

- ✅ Component picking from warehouse

## Post-Go-Live Grace Period (22-29 Dec)

- 📞 Support printing vendor integration

- 🔧 Adjust work order cycles if needed

- 📚 Train team on new printing workflow

Document Status: ✅ COMPLETE – Manufacturing Configuration Documented
 Prepared: 19 December 2025
 Next Review: 22 December 2025 (Go-Live Day)
 Manufacturing Workflow: Ready for Phase 1 + Printing Enhancement